LAB 5

COURSE: COMPUTER NETWORKS

# IPV4 ADDRESS AND NETWORK DEVICES

| Name: | ID: |
| --- | --- |

# Objective

Part 1: Convert IPv4 Addresses from Dotted Decimal to Binary

Part 2: Use Bitwise ANDing Operation to Determine Network Addresses

Part 3: Identify IPv4 Addresses and Apply Network Address Calculations

Part 4: Classify IPv4 Addresses

Part 5: Connect the network devices.

Part 6: Wireless configuration.

# Submitted file.

- Files to be submitted: Lab5a_ID.pka, Lab5b_ID.pka and Lab5_ID.pdf

- Name on the User profile must be changed to your ID.

Convert IPv4 Addresses from Dotted Decimal to Binary

Convert decimal numbers to their binary equivalent.

Fill in the following table by converting the decimal number to an 8-bit binary number. The first number has been completed for your reference. Recall that the eight binary bit values in an octet are based on the powers of 2, and from left to right are 128, 64, 32, 16, 8, 4, 2, and 1.

| Decimal | Binary |
| --- | --- |
| 192 | 11000000 |
| 168 | 10101000 |
| 10 | 00001010 |
| 255 | 11111111 |
| 2 | 00000010 |

Convert the IPv4 addresses to their binary equivalent.

An IPv4 address can be converted using the same technique you used above. Fill in the table below with the binary equivalent of the addresses provided. To make your answers easier to read, separate the binary octets with a period.

| Decimal | Binary |
| --- | --- |
| 192.168.10.10 | 11000000.10101000.00001010.00001010 |
| 172.16.18.183 | 10101100.0001000.00010010.10110111 |
| 10.86.252.17 | 00001010.01010110.11111100.00010001 |
| 255.255.255.128 | 11111111.11111111.11111111.10000000 |
| 255.255.192.0 | 11111111.11111111.11000000.00000000 |

Use Bitwise ANDing Operation to Determine Network Addresses

In Part 2, you will use the bitwise ANDing operation to calculate the network address for the provided host addresses. You will first need to convert an IPv4 decimal address and subnet mask to their binary equivalent. Once you have the binary form of the network address, convert it to its decimal form.

Note: The ANDing process compares the binary value in each bit position of the 32-bit host IP with the corresponding position in the 32-bit subnet mask. If there two 0s or a 0 and a 1, the ANDing result is 0. If there are two 1s, the result is a 1, as shown in the example here.

Determine the number of bits to use to calculate the network address.

| Description | Decimal | Binary |
| --- | --- | --- |
| IP Address | 192.168.10.131 | 11000000.10101000.00001010.10111111 |
| Subnet Mask | 255.255.255.192 | 11111111.11111111.11111111.11000000 |
| Network Address | 192.168.10.128 | 11000000.10101000.00001010.10000000 |

In the example above, how many bits are used to calculate the network address?

______________

Use the ANDing operation to determine the network address.

Enter the missing information into the table below:

| Description | Decimal | Binary |
| --- | --- | --- |
| IP Address | 172.16.145.29 | 10101100.00010000.10010001.00011101 |
| Subnet Mask | 255.255.0.0 | 11111111.11111111.00000000.00000000 |
| Network Address | 172.16.0.0 | 10101100.00010000.00000000.00000000 |

Enter the missing information into the table below:

| Description | Decimal | Binary |
| --- | --- | --- |
| IP Address | 192.168.10.10 | 11000000.10101000.00001010.00001010 |
| Subnet Mask | 255.255.255.0 | 11111111.11111111.11111111.00000000 |
| Network Address | 192.168.10.0 | 11000000.10101000.00001010.00000000 |

Apply Network Address Calculations

- Using command ipconfig to show the IP address of your computer and identify the IP address of your phone.

- What is IP address for your PC?  10.238.18.246

- What is the network address for your PC?  10.238.0.0

- What is IP address for your friend’s PC/your phone?  10.238.22.71

- What is the network address for your friend’s PC/your phone? 10.238.0.0

- Will your PCs and your phone be able to communicate directly with each other? N

- What is the default gateway address for your PC? 10.238.0.1

Classify IPv4 Addresses

Analyze the table shown below and identify the type of address (network, host, multicast, or broadcast address).

The first row shows an example of how the table should be completed.

- Network: all bit host = 0

- Broadcast: all bit host = 1

- Host: both 0 and 1

- Multicast: 224-239

| IP Address | Subnet Mask | Address Type |
| --- | --- | --- |
| 10.1.1.1 | 255.255.255.252 | Host |
| 192.168.33.63 | 255.255.255.192 | Broadcast |
| 239.192.1.100 | 255.252.0.0 | Multicast |
| 172.25.12.52 | 255.255.255.0 | Host |
| 10.255.0.0 | 255.0.0.0 | Network |
| 172.16.128.48 | 255.255.255.240 | Network |
| 209.165.202.159 | 255.255.255.224 | Broadcast |
| 172.16.0.255 | 255.255.0.0 | Host |

Analyze the table shown below and identify whether the address/prefix pair is a valid host address.

| IP Address/Prefix | Valid Host Address? | Reason |
| --- | --- | --- |
| 172.16.255.0/16 | Yes | Host Address |
| 192.31.7.255/24 | No | Broadcast |
| 64.102.255.255/14 | Yes | Host Address |
| 224.0.0.5/16 | No | Multicast |

Note: Before doing Part 5 and part 6, the students need to download and install Cisco Packet Tracer.

Students enroll the course “Packet Tracer” at and download Cisco packet tracer https://www.netacad.com/courses/packet-tracer

Connecting network devices

Open file: Lab5a- Connect a Wired and Wireless LAN.pka and work with the instruction inside the simulation file.

Connecting devices: The instruction gives the table of connection type as table below; students need to choose the right cable type to connect the interfaces.

| Device | Interface | Cable type | Connected device | Connects To |
| --- | --- | --- | --- | --- |
| Cloud | Eth6 | Copper Straight-Through | Router 0 | Fa0/0 |
|  | Coax7 | Coaxial | Cable Modem | Port0 |
| Cable Modem | Port0 | Coaxial | Cloud | Coax7 |
|  | Port1 | Copper Straight-Through | WirelessRouter | Internet |
| Router0 | Console | Console | Configuration Terminal | RS232 |
|  | Fa0/0 | Copper Straight-Through | Cloud | Eth6 |
|  | Fa0/1 | Copper Cross-Over | netacad.pka | Fa0 |
|  | Ser0/0/0 | Serial | Router1 | Ser0/0 |
| Router1 | Ser0/0 | Serial | Router0 | Ser0/0/0 |
|  | Fa1/0 | Fiber | Switch | Fa0/1 |
| WirelessRouter | Internet | Copper Straight-Through | Cable Modem | Port 1 |
|  | Eth1 | Copper Straight-Through | Family PC | Fa0 |

After finishing the lab, students save the file with filename: Lab5a_ID.pka

Wireless configuration

Open file: Lab5b - Configure a Wireless Network.pka and work with the instruction inside the simulation file.

After finishing the lab, students save the file with filename: Lab5b_ID.pka